Provider Selection and Task Allocation Problems

under Fuzzy QoS Constraints and Volume Discount Pricing Policy for Telecommunication Network

Hasan Hüseyin Turan1, Nihat Kasap2, Hüseyin Savran3

1University of Yalova, Department of Industrial Engineering, Yalova, 77100, TURKEY

2Sabancı University, School of Management, Istanbul, 34956, TURKEY

1University of Yalova, Department of Computer Engineering, Yalova, 77100, TURKEY

1 hturan@yalova.edu.tr, 2nihatk@sabanciuniv.edu, 3huseyinsavran2006@gmail.com

Abstract. Nowadays every firm uses telecommunication networks to continue their existence and complete their daily operations. In this paper, we investigate an optimization problem that a firm faces when acquiring network capacity from a market in which there exist several network providers offering different pricing and quality of service schemes. In the mathematical model, quality of service level guaranteed by network providers and minimum quality of service level which is needed to accomplishing operations are denoted as fuzzy numbers. Finally, a heuristic solution procedure is proposed to solve resulting nonlinear mixed integer programming model with fuzzy coefficients.

Keywords: telecommunication, fuzzy quality of service, heuristic algorithm

- 1   Introduction

Usage of data networks encompasses not only the traditional data applications but also newer applications such as real-time audio/video streaming, voice over TCP/IP, real-time transactions and other batch transactions over digital networks [1]. Firms in other words customers can lease or buy required amount of network resource from telecommunication network suppliers to perform their tasks. Each of these mentioned operations are assigned one of the acquired resources. Every task requires distinct amount of network capacity and different quality of service (QoS) requirements. As a consequence, not only cost but also QoS levels that are guarantied by network providers have to be taken into account by firms while selecting telecommunication network suppliers.

The QoS requirements for an operation include a wide range of parameters such as bandwidth, loss ratio, maximum delay and delay variation (jitter) [2]. Due to the non-deterministic nature of telecommunication network environment, it is not always possible to correctly estimate these parameters beforehand. Therefore, the main contribution and innovation of proposed work is the development of a mechanism for solving complex selection and task allocation problems in communication networks.  Based on the fact that accurate information needed for making these decisions is not available. In the QoS related problems, QoS requirements by the users can be regarded as fuzzy terms, since there exist the imprecision of fuzziness inherent in human judgments. Network managers often avoid specifying crisp values for their goal [3]. Moreover, the imprecision of network state seems to be always present and our best bet is to live with it [3].

In this paper, we investigate an optimization problem that a firm faces when acquiring network capacity from a market in which there exist several network providers offering different pricing and quality of service schemes. Quality of service level guaranteed by network providers and minimum quality of service level which is needed to accomplishing operations are denoted as fuzzy numbers. Moreover, proposed model has ability of coping with different price discount policies (volume discount pricing) that are offered by network providers in order to encourage customers to acquire large scale capacity. The problem is modeled from the firm’s (customer’s) point of view and assumes customer is rational decision maker. Although there exist some studies in current literature related to routing and infrastructure by taking into account fuzzy quality of service, best to our knowledge there is almost no study about how fuzzy quality of service constraints effect network capacity acquiring and task allocation policies. To sum up, the aim of this research is suggesting a novel mathematical model and developing efficient solution algorithm to the capacity acquisition and task allocation problem which is encountered by firms using network capacity.

2   Literature Review

Fuzzy logic has been applied to the several networking area during the last decade, and we believe that our study can extend the use of fuzzy logic in this area and particularly in task allocation. It is possible to divide current literature into two clusters that are related to application of fuzzy QoS parameters in telecommunication industry. One of these clusters might be considered as routing related researches under the assumption of fuzzy delay, jitter and package loss. Chen et al. [3] argues that it is unreasonable and inefficient to express the QoS constraints with deterministic crisp values. They presents new fuzzy genetic algorithm for QoS multicast routing and simulation experiments to demonstrate the algorithm’s efficiency. Cohen et al. [2] also tries to solve routing problem (path selection problem) under uncertainty. They present a fuzzy-based method for path selection under additive quality of service constraints, where the information available for making routing decisions is inaccurate. Guerin et al. [4] and Lorenz et al. [5] argue that the QoS constraints usually represented in the form of crisp limits. However, in actual network environment this kind of information is quite imprecision because of non-negligible propagation delay of state message, relative long update period due to overhead concern, and hierarchical state aggregation. Rai et al. [6] proposes a multi-objective Pareto-optimal technique using Genetic Algorithm (GA) for group communication, which determines a min-cost multicast tree satisfying end-to-end delay, jitter, packet loss rate and blocking probability constraints. Their model incorporates a fuzzy-based selection technique for initialization of QoS parameter values at each instance of multicasting.  Another existing set of literature about fuzzy QoS parameters mostly concentrates on selection or ranking of telecom suppliers or services under inaccurate or vague data.  Robak et al. [7] introduce modeling methodology of the QoS requirements for e-Business transactions between the service requestors and the service providers by usage of fuzzy methods. Wang [8] argues that service selection based on QoS-aware can bring great benefits to service consumers, as it is able to reduce redundancy in search.

This section of literature review is devoted to articles that are concerned with supplier selection under existence of fuzzy information. Önüt et al. [9] develop a supplier evaluation approach based on the analytic network process (ANP) and the technique for order performance by similarity to ideal solution (TOPSIS) methods to help a telecommunication company in the GSM sector in Turkey. Kumar et al. [10] presents fuzzy Multi-objective Integer Programming Vendor Selection Problem formulation that incorporates the three important goals: cost-minimization, quality-maximization and maximization of on-time-delivery-with the realistic constraints such as meeting the buyers’ demand, vendors’ capacity, vendors’ quota flexibility, etc. Amid et al. [11] develops a fuzzy multi objective model is formulated in such away as to simultaneously consider the imprecision of information and determine the order quantities to each supplier based on price breaks. Amid et al. [12] suggests a weighted max–min fuzzy model to handle effectively the vagueness of input data and different weights of criteria.

3   Problem Definition and Proposed Mixed Non-Linear Integer Mathematical Programming Model with Fuzzy QoS Constraints

In the model, it is assumed that all tasks and operations that have to be performed by firm can be divided into two discrete sets namely size fixed and time fixed tasks without loss of generality. It is also assumed that all mentioned tasks independently require different transmission time, bandwidth and quality of service level. In the mathematical model, QoS level guaranteed by network providers and minimum QoS level which is needed to accomplishing operations are denoted as triangular fuzzy numbers (TFN). Two important QoS factors, delay and jitter, are modeled by fuzzy membership functions. Figure 1 shows delay membership functions for two different telecommunication suppliers.

Objective function of presented model includes two cost terms, cost of acquiring capacity that reflects different pricing and discount schemes of suppliers and the potential opportunity costs such asthe cost of degradation in quality (which is more relevant for real time streaming applications), and the QoS switching cost when the decision maker intentionally downgrades the QoS level or switch to another supplier. The parameters define network providers are bandwidth, duration and QoS level.

Fig.1. Delay fuzzy membership functions of two bandwidth provider

Problem Notation:

I:Indexed set of suppliers

J:Indexed set of tasks

AT:Indexed set of time fixed tasks

AS: Indexed set of size fixed tasks

AT:	Indexed set of time fixed tasks

mi:	Number of the bandwidth level provided by ithsupplier

αik:	Transmission efficiency provided by ithsupplier at kthbandwidth level

βik:	Amount of bandwidth provided by ithsupplier at kthbandwidth level

cik:	Unit cost of buying bandwidth from ithsupplier at kthbandwidth level

Li:	Duration of contract ithsupplier

Ci:	Maximum amount of bandwidth provided by ithsupplier

cjo:	Opportunity cost for task j(j∈AT) not reaching target transmission rate

wj:	Size of task j(j∈AS) in terms of bits

rjU:	Maximum transmission rate for task j

rjL:	Minimum transmission rate for task j

Δtj:Expected transmission duration for task j (j∈AT)

σi,δi: Fuzzy triangular delay and jitter values provided by ithsupplier, respectively

μj,ηj: Fuzzy triangular delay and jitter values requested by taskjth, respectively

Decision variables:

rj: Transmission rate of jthtask

tj: Transmission starting of jthtask

xik=1, if kth bandwidth level is choosen from ith supplier

yij=1, if task jth is assigned to supplier i

yijt=1, if at time t task j is transmited by using supplier i

Mathematical Model:

Min i∈Ik=1micikxikβik+i∈Ij∈ATk=1mi(rjU−rjαik)cjoyij			(1)

Subject to:

j∈ASwjyij+j∈ATΔtjrjyijαikk=1mixikαikβikLi	∀i∈I			(2)

yijμj≥σi					∀i∈I,∀j∈J		(3)

yijηj≥δi					∀i∈I,∀j∈J		(4)

tj+Δtjyij≤Li				∀i∈I,∀j∈AT	 (5)

j∈Jrjyijt−k=1mixikαikβik≤0			∀i∈I,t=1,2,…,Li	(6)

i∈Iyij=1					∀j∈J			(7)

i∈It≤Liyijt=Δtj				∀j∈AT		(8)

i∈It≤Lirjyijt=wj				∀j∈AS		(9)

k=1mixik≤1					∀i∈I			(10)

yij−k=1mixik≤0				∀i∈I,∀j∈J		(11)

yijt≤yij					∀i∈I,∀j∈J,t≤Li	(12)

rjαik≥yijrjL					∀i∈I,∀j∈AT,k=1,..,mi(13)

rjαikyij≥rjU					∀i∈I,∀j∈AT,k=1,..,mi(14)

k=1mixikβikLiCi				∀i∈I			(15)

rj,tj≥0					∀j∈J			(16)

xik=0,1					∀i∈I,k=1,…,mi	(17)

yij=0,1					∀i∈I,∀j∈J		(18)

yijt=0,1					∀i∈I,∀j∈J,t≤Li	(19)

Above mathematical model’s objective function represents cost minimization problem for firms that buy capacity from telecommunication market. Objective function includes two cost terms. First expenditure term consists of total capacity acquiring cost by taking into account discounts.  Second term reflects opportunity cost because of not meeting target transmission rate. Constraint set (2) ensures that we can use only up to available capacity. Fuzzy constraint sets (3) and (4) guarantee that task can be assigned to only resources (suppliers) which provide lower or equal delay and jitter values, respectively. Constraint (5) ensures that all time fixed tasks assigned to a resource are completed when the resource is available. Constraint set (6) prevents using more bandwidth than available at any time. Constraint (7) together with (18) ensures that a task is allocated just only one resource and all tasks are allocated. (8) Numbered constraint set guarantees that all time fixed jobs actually placed into required amount of time slice. On the other hand, constraint (9) forces to reverse enough capacity for size fixed tasks. Constraint set (10) prevents buying more than one bandwidth level from any supplier. Next constraint (11) enforces assigning tasks only to resources that is already bought. Constraint set (12) ensures that a task is allocated a network resource only if it is occupies a time slice on it. Constraints (13) and (14) put limitation on possible minimum and maximum transmission rate of each time fixed task, respectively. Constraints set (15) ensure not acquiring capacity more than sold by network providers. Rest of the constraints is used for sign and binary restrictions

4   Solution Heuristic and Computational Study

The proposed model will turn into special case of well-known 2-Dimensional bin packing problem, if all tasks are considered as time fixed and opportunity cost term in objective function is removed. In addition, it is also necessity deleting fuzzy QoS constraints that are introduced in above formulation to make our model somehow similar to bin packing formulation.  2-Dimensional Bin packing problems are considered as NP Hard [13]. Therefore, it can be concluded that suggested cost optimization model for firms is also NP Hard in strong sense.  To sum up, existing commercial optimization software packages do not have ability of solving our model in reasonable amount of time. Hence, it is essential to develop fuzzy heuristic algorithm in order to solve existing problem. In the context of possibility theory, there are different approaches to model the coefficients of the objective function and/or the constraints as fuzzy members. Here the transformation method proposed by Gen et al.  [14] and used by [15] is adopted and extended to transform fuzzy right hand side numbers and fuzzy technological coefficients numbers in greater than or equal type constraints to crisp constraints.

j=1naijx≥bi		i=1,⋯,m					(20)

where xjis the j-th decision variable; aijis the fuzzy technological coefficient of the i-th constraint and j-th decision variable; and biis the fuzzy right hand side term of i-th constraint. Crisp formulation of (20) is given below.

j=1n((1−λ)aij1+λaij2)x≥(1−λ)bi1+λbi2	i=1,⋯,m			(21) where 0≤λ≤1is a cutoff value that has to be established parametrically. Therefore, the transformation of the fuzzy QoS constraints sets (3) and (4) in Section 3 into crisp inequalities are performed by using (21).  In order to solve the problem λis settled down parametrically to obtain the value of objective function (z) for each one of thoseλ∈0,1. The result is, however, a fuzzy set planner has to decide which pair (λ,z) considers the optimal if he wants to obtain a crisp solution.

Fuzzy Heuristic Algorithm:

Step 1: For each task j∈Jcalculate delay equivalent of taskμjand jitter equivalent of taskηj, for given value of λby using (21). Afterwards, sort all tasks first increasing order of μjthen increasing order ofηj.

Step 2: Sort the list by decreasing order of sizes of tasks. Call this listJ.

Step 3: For each supplier i∈Icalculate delay equivalent of resourceσi and jitter equivalent of resourceδi for given a given value of λ by using (21).

Step 4: Sort all suppliers first increasing order of σithen increasing order ofδi.

Step 5: Sort the list obtained at Step 4 by increase order of effective unit capacity costcik. Where cik=cikLiαik. Call this listI.

Step 6: Letrj=rjU, j∈AT. Starting from the first element of Jassign tasks to suppliers (bins) again starting from the top ranked supplier of Ias long as μj≥σiand ηj≥δiholds simultaneously

Step 7: After allocation of all tasks, find least used most expensive supplier. Try to move all tasks in this bin into other already chosen bins by taking opportunity cost and swapping tasks among resources. Calculate the current total cost by Eq. (1)

Step 8: Repeat Step 7, if it is feasible and economical, otherwise stop.

Table 1 shows the results of a test case where firm has to complete 100 tasks and also there exists 40 different suppliers in the market. Test scenario analyzes indicates that changing value of λdoes not effect the number of supplier selected. Yet, it effects which supplier is selected. Optimal Solution for decision maker is observed when λis chosen 0.5.

Table 1 Test results for changing λ values

| # of Suppliers | # of Tasks | λ | Total Cost | Opportunity cost | Capacity purchasing cost | # Supplier Selected | Unused Capacity (%) | Run Time (sec.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 40 40 40 40 40 40 40 40 40 40 | 100 100 100 100 100 100 100 100 100 100 100 | 0 0.1 0.2 0.3 0.4 0.5 0.6 0.7 0.8 0.9 1 | 288.375 288.883 288.725 289.195 290.204 286.100 286.547 287.571 287.558 288.690 287.752 | 41.7375 42.2456 42.0878 42.5576 55.1896 39.4625 39.9097 40.9396 40.9205 42.0530 52.7371 | 246.637 246.637 246.637 246.637 235.015 246.637 246.637 246.637 246.637 246.637 235.015 | 5 5 5 5 5 5 5 5 5 5 5 | 0.12 0.26 0,19 0,26 0,05 0,03 0,16 0,25 0,36 0,39 0,002 | 3.939 3.699 3.483 3.858 4.965 3.912 3.919 4.190 3.929 3.945 4.934 |

Acknowledgements

This research is supported by the Scientific and Technological Research Council of Turkey (TÜBİTAK), Short-Term R&D Funding Program (Grant No. 110 K 609).

References

- Kasap, N., Aytuğ, H., and Erengüç, S. S. (2007). Provider Selection and Task Allocation Issues in Networks with Different QoS Levels and All You Can Send Pricining, Decision Support Systems, 43(2), pp.375-389.

- Cohen, A., Korach, E., Last, M., Ohayan, R. (2005). A fuzzy-based path ordering algorithm for QoS routing in non-deterministic communication networks, Fuzzy Sets and Systems, 150, pp.401-417.

- Chen, P., Dong, T. (2003). A fuzzy genetic algorithm for QoS multicast routing, Computer Communications, 26, pp.506-512.

- Guerin, R., Orda, A. (1999). QoS-based routing in networks with inaccurate information: theory and algorithms, IEEE/ACM Trans. Networking, 7 (3), pp.350–364.

- Lorenz, D., Orda, A. (1998). QoS-based routing in networks with uncertain parameters, IEEE/ACM Trans. Networking, 6 (6), pp.768–778.

- Rai, S., C., Misra, B., B., Nayak, A., K., Mall, R., Pradhan, S., K. (2010). A Multi-Objective QoS optimization with fuzzy based parameter setting for real-time multicasting, Int. J. Communications, Network and System Sciences, 3, pp.530-539.

- Robak, S., Pieczynski, A. (2008). Fuzzy modeling of QoS for e-business transactions realized by web services, Journal of Applied Computer Science, 16(1), pp.69-79.

- Wang, P. (2009). QoS-aware web services selection with intuitionistic fuzzy set under consumer’s vague perception, Expert Systems with Applications, 36, pp.4460-4466.

- Önüt, S., Kara, S. S., Işık, E. (2009). Long term supplier selection using a combined fuzzy MCDM approach: A case study for a telecommunication company, Expert Systems with Applications, 36, pp.3887-3895.

- Kumar, M., Vrat, P., Shankar, R. (2006). A fuzzy programming approach for vendor selection problem in a supply chain, Int. J. Production Economics, 101, pp.273-285.

- Amid, A., Ghodsypour, S., H., Q'Brein, C. (2009), A weighted additive fuzzy multi objective model for the supplier selection problem under price breaks in a supply Chain, Int. J. Production Economics, 121, pp.323-332.

- Amid, A., Ghodsypour, S., H., Q'Brein (2011). A weighted max–min model for fuzzy multi-objective supplier selection in a supply chain, Int. J. Production Economics, 131, pp.139-145.

- Martello, S., Vigo, D., (1998). Exact solution of the two-dimensional finite bin packing problem, Management Science, 44(3), pp.388-399.

- Gen, M., Tsujimura,Y., Ida, K., (1992). Method for solving multi objective aggregate production planning problem with fuzzy parameters. Computers and Industrial Engineering, 23, pp.117–120.

- Mula, J., Peidro, D., Poler, R., (2010). The effectiveness of a fuzzy mathematical programming approach for supply chain production planning with fuzzy demand. . International Journal of Production Economics, 128, pp.136–143.